Advancing Non-Destructive Evaluation in Additive Manufacturing: A Micro-Computed Tomography Protocol for Cold Spray

Tumulu Satish Kumar1, Tyler Dolmetsch2, Tanaji Paul2 and Arvind Agarwal1

- Cold spray for extreme environments (Cold - X) Laboratory, Department of Mechanical and Nuclear Engineering, Virginia Commonwealth University, Richmond, VA 23220, United States

- Cold Spray and Rapid Deposition (ColRAD) Laboratory, Department of Mechanical and Materials Engineering, Florida International University, 10555 West Flagler Street, Miami, FL 33174, United States

Across metal additive manufacturing (AM), high-resolution X-ray computed tomography (CT) is recognized as a key tool for rapid nondestructive evaluation (NDE) and process qualification but faces challenges such as beam-hardening artifacts, long scan times, and high cost for dense or large components. No established micro-CT standards exist for cold spray (CS) regarding voxel size, scan energy, region-of-interest selection, or segmentation thresholds relative to component thickness and alloy type. Hence, the application of micro-CT in CS is still limited and not standardized, although it has proven to be highly effective in visualizing and quantifying porosity and internal defects in AM-ed metallic components. Inter-laboratory validation and reproducibility studies are also scarce. Earlier X-ray micro-CT studies on cold-sprayed components reveal strong sensitivity of deposit porosity to process variables, valuable for design and process optimization. This work aims to define recommended micro-CT parameters (energy, filtration, voxel size, and exposure), specify maximum inspectable thicknesses for given energies, and establish validated image-processing and porosity quantification protocols tailored for typical CS structures. Bulk copper samples produced by CS are examined using X-ray micro-CT to systematically evaluate internal porosity. The study focuses on correlating the process parameters with the material density and defects to establish reliable inspection and quantification guidelines for cold-sprayed copper deposits.